Vehicle Extrication

Chapter 3: Mechanical Energy and Vehicle Anatomy

Ready for Review

- Three concepts of energy are typically associated with injury: potential energy, kinetic energy, and work.

- Motor vehicle collisions are classified traditionally by the area of initial impact: front impact (head-on), lateral impact (T-bone, side impact), rear-end, rotational (spins), and rollovers.

- In every crash three collisions occur:

  - The collision of the vehicle against an object

  - The collision of the passenger against the interior of the vehicle

  - The collision of the passenger’s internal organs against the solid structures of the body

- Before a rescuer can properly apply any extrication procedures to a vehicle, he or she must understand the inner and outer components that make up a vehicle system.

- The Department of Transportation (DOT) classifies vehicles based on whether the vehicle transports passengers or commodities, with a nonpassenger vehicle being further classified by the number of axles and unit attachments it has.

- The Department of Energy (DOE) classifies vehicles by size utilizing a cubic foot system (passenger and cargo volume) and gross weight system.

- Most vehicles on the road today are conventional-type vehicles; these types of vehicles utilize internal combustion engines (ICEs) for power. Other types of vehicles include hybrid electric vehicles, hydrogen fuel cell vehicles, and electric-powered vehicles.

- Electrical power in conventional-type vehicles with internal combustion engines is supplied by a basic 12-volt lead acid battery system. In hybrids, fuel cell vehicles, and electric vehicles, a different, advanced electrical design is used.

- The development of strong, crash-resistant vehicles requires engineers and the steel industry to develop stronger and lighter steels to meet demands.

- There are two frame systems that are most common in today’s vehicles: body-over-frame construction and unitized or unibody construction. These frames can be composed of steel (most common), aluminum, or carbon fiber/composite. Another type of frame system that is less common today is the space frame, which can consist of aluminum construction or tubular steel construction.

- Several key components make up the body portion of the vehicle.

- The technical rescuer can encounter several types of glass in a vehicle, including laminated safety glass, tempered safety glass, enhanced protective glass (EPG), polycarbonate, and ballistic glass.