Vehicle Extrication

Chapter 5: Supplemental Restraint Systems

Ready for Review

- In 1967 the National Highway Traffic Safety Administration (NHTSA) issued a federal mandate titled the Federal Motor Vehicle Safety Standard 209 (FMVSS 209), Seatbelt Assemblies. This was the first of many subsequent regulations that outlined minimum safety requirements for motor vehicles mandating compliance from vehicle manufacturers.

- In the 1980s, the air bag became known as a supplemental restraint system (SRS) by working in conjunction with the seat belt.

- Manual seat belts are classified as an active restraint device because the occupant has to activate the system by engaging the seat belt mechanism into the anchor unit.

- A vehicle air bag is classified as a passive restraint device because the occupant does not have to activate the device to make it function; the system is automatically activated when power is applied to the vehicle.

- Smart air bag systems are using adaptive response features such as dual-stage or multistage inflation and suppression systems.

- A four-stage process occurs when an air bag deploys in a crash sequence: the crash itself, the crash sensor detecting deceleration, the air bag deploying and inflating, and the occupant moving forward and striking the bag as deflation occurs.

- Several components make up an air bag, including the air bag, initiator, electronic control unit, inflator, and sensors.

- Roll-over protection systems (ROPS) were initially designed for convertible vehicles to protect occupants in vehicle roll-over incidents. Roll bars are concealed until activated by sensors.

- Seat belt assemblies can be housed in any post or column, under the seats, or in the center console. If cutting through a post at the bottom or top, the molding must be removed to reveal the pretensioning system in order to cut around the device.

- Never assume that an air bag is dead just because the power has been disconnected; a vehicle air bag system comes equipped with an energy capacitor, which can store power for up to 30 minutes in some models.

- Eliminating potential hazards of SRS systems may include disconnecting power, recognizing and identifying air bags, distancing, and following certain rules.