Vehicle Extrication

Chapter 7: Site Operations

Ready for Review

- Site operations, vehicle stabilization, and victim management are parts of a successive three-phase process in vehicle extrication.

- Ensuring that proper safety procedures are followed in any operation is paramount for any organization.

- Because many injured patients require definitive care in less than an hour, the “Golden Hour” is now commonly referred to as the “Golden Period.”

- Before the technical rescuer can start the operational process, he or she must be operationally ready, not only in the area of mental and physical readiness, but particularly in the area of personal protection.

- The protective ensemble includes the body piece, helmet, primary eye protection, gloves, footwear, and oftentimes reflective safety vests and respiratory protection.

- Per the federally mandated National Incident Management System (NIMS), common language is the goal for greater continuity and interoperability between agencies.

- The main goal of defensive apparatus placement is to block and protect the scene from the flow of traffic.

- Crowd control is an absolute necessity in protecting responders from individuals who might attempt to enter into the emergency site.

- As an incident grows, the need for additional resources and personnel can grow as well.

- A properly equipped rehabilitation unit providing shelter and thermal control options should be established early during prolonged extrication incidents.

- When responding to vehicle and machinery incidents, it is vital to know what additional specialized equipment is available to assist in managing an incident.

- Proper documentation always pays dividends for an agency when legal issues such as lawsuits occur.

- Size-up begins at the time the incident is dispatched, not at the time the unit arrives at the scene.

- A size-up report gives an update to the units responding so they can organize their response.

- There are generally three categories of entrapment: minor, moderate, and heavy.

- The inner and outer surveys are 360-degree inspections of the scene that are completed by two personnel.

- An incident action plan (IAP) may be formal or informal.

- Scene safety zones or operational zones are divided into hot, warm, and cold zones. These zones are strictly enforced by a designated incident safety officer.

- Hazards at vehicle extrication incidents may include fire hazards, electrical hazards, fuel hazards, ignition sources, and hazardous materials. Personnel should possess the necessary skills, training, and qualifications before attempting to mitigate any of these hazards.

- You should be familiar with the capabilities, protocols, and methods for accessing and landing helicopters in your area.