Vehicle Extrication

Chapter 10: Alternative Extrication Techniques

Ready for Review

- Tunneling is the process of gaining entry through the rear trunk area of a vehicle. It is most commonly performed for a vehicle that is resting on its roof, but it can be performed for a vehicle in any resting position.

- There are multiple variations of the types of seat frames.

- Seats are attached to the floorboards of the vehicle in four or more places, with positional adjustments on slide tracks that are either fully automatic/motorized or manually operated.

- The material covering the seats may have to be removed using a knife or trauma shears to expose the area of attachment located on the floor or to expose the steel hinges where the seat back is attached to the bottom seat section.

- The cross ram technique is used when the impingement of metal on the victim requires a unique mechanism of movement.

- The hydraulic ram has the advantage over a hydraulic spreader because it can be used in tight spaces.

- When an object impales a vehicle occupant, the main objective is to stabilize the penetrating object at the entry point and the exit point, if an exit point exists.

- The pneumatic air chisel is a powerful tool with multiple blades. The two most widely used blades are the panel cutter, or T-blade, and the flat/curved blade. The panel cutter is used for cutting shallow straight cuts on small-gauge sheet metal, and the flat/curved blade is used for cutting through medium- to heavy-gauge steel.

- A reciprocating saw is an excellent tool to use for removing a vehicle’s roof, especially on vehicles with large C-posts.

- The technique of removing a door on the hinge side can be completed using a combination of a hydraulic spreader and hydraulic cutter, or it can be accomplished utilizing an air chisel.

- Gaining entry through the side of a vehicle that has rolled over and is resting on its roof can require either a very basic procedure such as forcing a door open, or it can be a very involved process requiring multiple steps.

- Acceleration and brake pedals are notorious for trapping or entangling occupants’ foot extremities. The pedal arm can be cut and/or relocated using mechanical tools such as a hydraulic cutter, reciprocating saw, air chisel, or pneumatic cutoff tool (whizzer saw); a come along and chain set; or a very simple tool such as a section of webbing or rope.